Steps and procedure to run the Connector along with Simulator

Mobile App (BLE Serial Monitor) – Usage Steps

- Install the BLE Serial Monitor mobile application.

- Open the app and click on the menu list. You will see the name Devices, click on it.

- Select Bluetooth LE, then click on Scan.

  - You should find “NitaraCLV4” in the scanned device list.

  - If the device is not visible, it indicates that the firmware may not be loaded properly or the device is malfunctioning.

- Select NitaraCLV4. The terminal screen will appear.

- On the right side of the terminal, click on the Connect icon.

  - The status should change to Connected.

- Send the MA configuration command to the board.

  - After sending the command, you should receive a success response.

- After successful configuration, proceed to run the simulator.

Example MA Configuration Command:

{"command":"ma_port_config","model":1002,"baud_rate":1200,"data_bits":8,"stop_bits":1,"parity":0,"stream":0}#

Note:

- The model and baud rate must match the simulator settings.

Steps to Run the Connector Using the Simulator

- Connect the DB9 connector cable to the MA_IN port of the connector and connect the USB cable to the laptop.

- Launch the Simulator Software.

- In the simulator, select the MA port from the dropdown list.

  - If the port is not visible, open Device Manager, click Refresh Ports, and then select the correct port.

- Select the model and baud rate.
Note: The model and baud rate must match the configuration sent from the mobile app via BLE.

- Click on Connect MA, you will get the connected and port on the log.

- Go to the Controls tab. In the Both field, click Send Normal.

- After approximately 5 seconds, click the Stop All button.

- The simulated MA data should be received and displayed on the mobile app (BLE Serial Monitor).